# NIRS GUI User Manual

A Graphical Interface for Processing Near-Infrared Spectroscopy Data

Author: Michael Wolfe

Version: 1.0

Last Updated: April 30, 2025

# 1. Introduction

This manual guides users through the features and usage of the NIRS GUI. The GUI simplifies preprocessing, visualization, and asymptotic regression modeling of NIRS data collected during chemotherapy treatment cycles.

# 2. System Requirements

- R (version ≥ 4.2)
- RStudio (required)
- Required R packages: shiny, ggplot2, data.table, dplyr, DT, devtools, and the custom NIRS package (e.g., nirsdexata)
- Mac or Windows OS

# 3. Launching the Application

Step 1: Install the required packages (once)

install.packages(c("shiny", "ggplot2", "data.table", "dplyr", "DT", "devtools"))

Step 2: Load your custom NIRS package

devtools::load_all("path/to/nirsdexata")

Step 3: Run the app

shiny::runApp("path/to/your/app/GUI.R")

# 4. Interface Overview

The GUI contains the following tabs:

## a. Cycle Processing Tab

- Upload Single Cycle File: Upload .txt NIRS data file.
- Subject ID & Cycle Number: Enter identifiers manually.
- Start Time: Input minutes and seconds to mark the start of the measurement window.
- Drop Wavelength: Optionally remove specific wavelengths (e.g., TSI) before regression.
- Apply Cycle Settings: Sets metadata and preprocessing options.
- Tau Visualization Dashboard: Displays regression output by rest order and chromophore.
- Done Button: Saves summary data (HbO only) to a summary object.

## b. Summary Table Tab

- Displays the collected tau values from each processed cycle.
- Allows export as .csv.

# 5. Functional Details

## Preprocessing Workflow

1. Uploaded data is parsed using read_nirs().
2. Measurement window is defined using start and end time (default: 16 minutes).
3. Optionally removed wavelengths are excluded before fitting.

## Asymptotic Regression

- Model: Asym - (Asym - R0) * exp(-exp(lrc) * time)
- Separate fits for each chromophore and rest order.
- Only HbO tau averages are included in exported summary.

## Data Export

- Use the “Export Summary” button to download cleaned and processed data.

# 6. Troubleshooting

Issue | Solution
------|--------
App crashes after clicking Done | Ensure that start time and subject ID are filled.
Tau values do not update | Confirm that the wavelength removal and Apply buttons are used before fitting.
"object 'steadystates' not found" | Verify the custom package is loaded with devtools::load_all()

# 7. Best Practices

- Use consistent subject and cycle naming (e.g., ID_3, Cycle 0).
- Always apply settings before clicking Done.
- Confirm removed wavelengths(usually TSI) before finalizing analysis.